This file is Public Domain and is to be distributed only as this intact

.lzh file, and all accompanying files must be included.

FASTcheck/CHIPcheck

HOW IT WORKS

Tests every memory location (Private and Public) for bad cells.

(NOTE:  That FASTROM on the Amiga 3000 will NOT be checked since that ram

is Write-Protected.  But if the system crashes on boot after installing

new RAM then it could be a bad location in the FASTROM).

Each long word that is scanned is cleared to zero and verified, and then

stored with $FFFF FFFF and verified.  If the ULONG does not equal the

verified values when read it displays the address , the zero-value, and the

FF-value for the address.

e.g.   $C00040    $00000000    $FFFFFFFD

Since the 680x0 stores in high word/high byte format the actual byte

addresses are as follows:

$C00040  bits 24-31 (highest byte)             $FF == %1111 1111

$C00041  bits 16-23 (second highest byte)      $FF == %1111 1111

$C00042  bits 8-15  (second lowest byte)       $FF == %1111 1111

$C00043  bits 0-7   (lowest byte)              $FD == %1111 1101

Which means that at $C00043 bit 1 is clear and won't set.

If a bad bit occurs the best option is to remove the faulty chip and

replace it with a good one (retesting the memory for bad bits).

HOW TO WORK IT

To check FAST-RAM enter FASTcheck from a CLI and the program installs itself

in CHIP-RAM where it is safe.  For help enter FASTcheck -h, and if you wish

to check a specific range enter:

FASTcheck [lo-address (hex)] [hi-address (hex)]

To check CHIP-RAM you MUST have FAST-RAM available (that's where CHIPcheck

gets loaded).  The commands for CHIPcheck are the same as FASTcheck:

CHIPcheck [-h] [lo-address (hex)] [hi-address (hex)]

IF IT DOESN'T WORK

If your system will not boot after installing new RAMs (after first

checking to be sure you seated them properly and in the right socket),

then a bad bit may exist in the Private Memory allocated for the system's

use.  i.e. User/System Stacks, Library Structures, Memory Lists, etc.

If you do not have other RAM chips available to replace those just put in,

then seek technical support from the Expansion board's manufacturer or

Commodore.

For any comments or suggestions, please write to me:

Charles Sultana

2871 Joanna

Farmers Branch, TX 75234